# Network-1 Project Documentation Report

Project Number: 5
Project Name : The Distributed E-Commerce Fulfillment Network
Discussion Time : 12:15

# Full Topology:

The network is designed as a Wide Area Network (WAN) consisting of four core routers (Fulfillment Center, Shipping Hub, Data Center, Corporate HQ) plus an ISP router. The topology includes switches, end devices (PCs), a DNS server, and a web server. All devices are interconnected via serial and Ethernet links, and static routing is used to achieve full connectivity. Screenshot: Full logical topology from Packet Tracer:

## Device & Connection Specifications:

Details of all hardware and physical-layer components used in the topology.

### Hardware Inventory

Device Type; Model; Quantity; Notes. Router; 2911; 3. Router; ISR4321; 2; Used instead of 2911 for implementing VPN. Switch; 2950-T; 7. PC / End Host; ----------; 98. Server (DNS); ----------; 1. Server (Web); ----------; 1.

### Cabling & Connection Types

Connection (From → To); Cable Type. Between Routers; Serial. Router and Switch; Copper Straight-Through. Between Switches; Copper Cross-Over. Switch and PCs(or Servers); Copper Straight-Through.

# successful ping between networks:

PC (Fulfillment Center) to Web Server (Data Center):

PC (Shipping Hub) to PC (HQ):

Any PC to ISP Router:

## 1. VLSM Chart  (Internal Networks)

The private IP range 172.16.0.0/16 was divided using VLSM to meet host requirements of each LAN. The following tables summarize the subnet allocation for internal networks and WAN links.

| Network Name | Hosts | Subnet | Network ID | Subnet Mask | First useable IP | Last useable IP | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fulfillment Center LAN | 81 | /25 (126 hosts) | 172.16.0.0 | 255.255.255.128 | 172.16.0.1 | 172.16.0.126 | 172.16.0.127 |
| Shipping Hub LAN | 15 | /27 (30 hosts) | 172.16.0.128 | 255.255.255.224 | 172.16.0.129 | 172.16.0.158 | 172.16.0.159 |
| Data Center LAN | 3 | /29 (6 hosts) | 172.16.0.160 | 255.255.255.248 | 172.16.0.161 | 172.16.0.166 | 172.16.0.167 |
| HQ VLAN 10 | 3 | /29 (6 hosts) | 172.16.0.168 | 255.255.255.248 | 172.16.0.169 | 172.16.0.174 | 172.16.0.175 |
| HQ VLAN 20 | 3 | /29 (6 hosts) | 172.16.0.176 | 255.255.255.248 | 172.16.0.177 | 172.16.0.182 | 172.16.0.183 |

## 2. VLSM Chart  (WAN Links)

| Link | Network ID | Subnet Mask | First useable IP | Last useable IP | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Data Center ↔ Corp HQ | 203.0.113.0 | 255.255.255.252 | 203.0.113.1 | 203.0.113.2 | 203.0.113.3 |
| Data Center ↔ Fulfillment | 203.0.113.4 | 255.255.255.252 | 203.0.113.5 | 203.0.113.6 | 203.0.113.7 |
| Data Center ↔ Shipping Hub | 203.0.113.8 | 255.255.255.252 | 203.0.113.9 | 203.0.113.10 | 203.0.113.11 |
| Data Center ↔ ISP | 203.0.113.12 | 255.255.255.252 | 203.0.113.13 | 203.0.113.14 | 203.0.113.15 |

## Router IP configurations:

## DHCP + sample PC:

DHCP pools were configured on the routers to automatically assign IP addresses to hosts in their respective LANs:

| Router | Pool Name | Network | Gateway | DNS Server |
| --- | --- | --- | --- | --- |
| Fulfillment | LAN1 | 172.16.0.0/25 | 172.16.0.1 | 172.16.0.162 |
| Shipping Hub | Shipping-Hub | 172.16.0.128/27 | 172.16.0.129 | 172.16.0.162 |
| Corporate HQ | Vlan1 | 172.16.0.168/29 | 172.16.0.169 | 172.16.0.162 |
| Corporate HQ | Vlan2 | 172.16.0.176/29 | 172.16.0.177 | 172.16.0.162 |

## VLAN table:

On the HQ switch, two VLANs were created: VLAN 10 (IT) and VLAN 20 (Accounting). Access ports were assigned accordingly:

VLAN ID; VLAN Name State Ports. 10; IT Active f0/1, f0/2. 20; Accounting Active f0/3, f0/4.

## NAT translation table:

NAT was implemented on the Data Center router

| Detail | Value |
| --- | --- |
| Router performing NAT | Data Center |
| NAT Type (Static / Dynamic / PAT) | Static for Web Server / PAT for End devices |
| Inside Interface(s) | Gig0/0/0 (Static) / Se0/1/0, Se0/1/1, Se0/2/0 |
| Outside Interface(s) | Se0/2/1 |
| NAT Pool / Overload Address | 203.0.113.13(Overload) |

## VPN status + successful ping:

A site-to-site IPsec VPN tunnel was established between the Fulfillment Center router and the Data Center router.

### VPN (Site-to-Site)

| Detail | Value |
| --- | --- |
| VPN Tunnel Start (Router) | Fullfilment Center |
| VPN Tunnel End (Router) | Data Center |
| Interesting Traffic (ACLs) | 172.16.0.0/25 and 172.16.0.160 /29 |
| Pre-shared Key | helwan |

## Routing table from each router:

Static routes were configured on every router to ensure reachability between all networks. Default routes were used on the branch routers pointing to the Data Center core router.

## DNS & WEB Servers :

### DNS Server

| Detail | Value |
| --- | --- |
| DNS Server IP Address | 172.16.0.162 |
| Domain Name configured | ecommerce.com |
| Points to (Web Server IP) | 172.16.0.163 |

## ICMP Testing:

Connectivity between all networks was verified using ICMP (ping). A successful ping from PC1 (Fulfillment Center, 172.16.0.6) to Web Server (172.16.0.163) confirms end-to-end reachability across the WAN.

Successful ping: Reply from 172.16.0.163 with 0% loss

Simulation mode: Packet path traced through routers, confirming static routes and NAT are working correctly.

#### Thank you, Eng. Ibrahim, for reading all the Documentation to the end 😊.